PCB DESIGN THEORY

Comprehensive Study Note

Printed Circuit Board Design — Principles, Materials, Layout, Signal Integrity & Manufacturing

Prepared for: Electronic & Embedded Systems Design Study

# Table of Contents

# 1. Introduction to PCBs

A Printed Circuit Board (PCB) is a laminated structure of insulating substrate and conductive copper layers that mechanically supports and electrically interconnects electronic components using conductive tracks (traces), pads, and vias etched from copper sheets. PCBs replaced point-to-point wiring, giving reliable, repeatable, compact and cost-effective interconnection for mass-produced electronics.

## 1.1 Why PCBs are used

- Mechanical support and fixed component positioning.

- Reliable, repeatable electrical connections (no loose wiring).

- Compact, lightweight, and suitable for automated assembly (SMT/THT).

- Controlled electrical characteristics — impedance, capacitance, inductance can be engineered.

- Better heat dissipation via copper planes and thermal vias.

- Reduced EMI when designed with proper grounding and layer stack-up.

## 1.2 Basic PCB terminology

| Term | Meaning |
| --- | --- |
| Substrate | Insulating base material (e.g., FR4) that supports copper layers |
| Trace / Track | Copper conductor carrying electrical signal or power |
| Pad | Copper area where a component lead is soldered |
| Via | Plated hole connecting copper on different layers |
| Footprint / Land pattern | Arrangement of pads matching a component package |
| Silkscreen | Printed layer showing labels, reference designators, polarity marks |
| Solder mask | Protective lacquer layer preventing solder bridging, except at exposed pads |
| Copper weight | Thickness of copper layer, in oz/ft² (1 oz ≈ 35 µm) |
| Annular ring | Copper ring around a drilled hole in a pad/via |
| Stack-up | Arrangement of copper and dielectric layers through board thickness |

# 2. Types of PCBs

## 2.1 By number of layers

- Single-sided (single layer) — copper on one side only; simplest, cheapest; used in low-density, low-cost products (chargers, simple power supplies).

- Double-sided (two layer) — copper on both sides connected by vias; moderate complexity; common in consumer electronics.

- Multilayer (4, 6, 8...layers) — alternating copper and insulating layers laminated together; needed for complex, high-density, or high-speed designs (dedicated power/ground planes, controlled impedance).

## 2.2 By substrate flexibility

| Type | Description | Typical use |
| --- | --- | --- |
| Rigid PCB | Fixed, non-bendable FR4-based board | General electronics, computers |
| Flexible PCB (Flex) | Polyimide-based, can bend/fold | Wearables, cameras, connectors in tight spaces |
| Rigid-flex PCB | Combination of rigid sections joined by flexible sections | Aerospace, medical devices, foldable products |

## 2.3 By application / material

- High-frequency/RF PCBs — low-loss substrates (Rogers, PTFE) for microwave/RF circuits.

- High-Tg / High-Temperature PCBs — for automotive, industrial environments.

- Metal-core PCB (MCPCB) — aluminium or copper base for high thermal dissipation (LED lighting, power electronics).

- HDI (High-Density Interconnect) PCB — microvias, fine pitch traces for compact high-pin-count devices (smartphones).

# 3. PCB Materials

## 3.1 Substrate (base laminate)

The substrate provides mechanical strength and electrical insulation between conductive layers. It is characterised by its dielectric constant (Dk / εr), dissipation factor (Df), glass transition temperature (Tg), and thermal expansion (CTE).

| Material | Dk (approx.) | Notes |
| --- | --- | --- |
| FR4 (glass-reinforced epoxy) | 4.2 – 4.8 | Most common; general purpose, good mechanical strength |
| Polyimide | 3.4 | Flexible PCBs, high temperature tolerance |
| Rogers / PTFE (Teflon) laminates | 2.2 – 3.5 | Low-loss, used for RF/microwave circuits |
| CEM-1 / CEM-3 | 4.5 approx. | Lower cost alternative to FR4, less mechanical strength |
| Aluminium / Metal-core | — | High thermal conductivity for power/LED applications |

## 3.2 Copper foil

Copper is the conductive layer, laminated onto the substrate. Its thickness is specified as copper weight in ounces per square foot (oz/ft²):

| Copper weight | Thickness (approx.) | Typical use |
| --- | --- | --- |
| 0.5 oz | 17.5 µm | Fine-pitch signal layers |
| 1 oz | 35 µm | Standard signal layers |
| 2 oz | 70 µm | Power planes, higher current traces |
| 3 oz+ | 105 µm+ | High-current / power electronics boards |

## 3.3 Prepreg and core

- Core — a fully cured laminate with copper foil already bonded on both sides.

- Prepreg — glass fabric pre-impregnated with uncured (B-stage) resin, used as the bonding/insulating layer between cores when pressing a multilayer stack; it cures under heat and pressure during lamination.

## 3.4 Surface finishes

Applied over exposed copper pads to protect against oxidation and provide a solderable surface:

| Finish | Full name | Notes |
| --- | --- | --- |
| HASL | Hot Air Solder Levelling | Low cost, uneven surface, not ideal for fine-pitch |
| ENIG | Electroless Nickel Immersion Gold | Flat surface, good for fine-pitch/BGA, more expensive |
| OSP | Organic Solderability Preservative | Cheap, flat, but shorter shelf life |
| Immersion Silver / Tin | — | Flat, good conductivity, tarnish/whisker concerns |

# 4. PCB Design Flow

A typical PCB design process moves from concept to manufacturable output through the following stages:

- Requirement/specification capture — functional, electrical, mechanical, thermal, EMI/EMC and regulatory requirements.

- Schematic capture — draw the circuit using a schematic editor, assign component symbols and footprints.

- Component selection & footprint verification — check package, tolerances, availability, cost.

- Board outline & mechanical constraints — define board shape, mounting holes, connector positions, enclosure fit.

- Layer stack-up definition — number of layers, copper weights, dielectric thickness, impedance targets.

- Component placement — arrange components for signal flow, thermal, and manufacturability.

- Routing (trace layout) — connect nets following design rules (widths, spacing, impedance).

- Design Rule Check (DRC) & Electrical Rule Check (ERC) — verify against manufacturing and connectivity rules.

- Generate outputs — Gerber files, drill files, pick-and-place files, Bill of Materials (BOM), assembly drawings.

- Design for Manufacturing (DFM) / Design for Assembly (DFA) review.

- Prototype fabrication, assembly, and testing (bring-up, functional test).

- Design iteration based on test results before production release.

Exam tip: Examiners often ask you to list this flow in order — remember the acronym pattern: Schematic → Placement → Routing → DRC → Gerber/Fab output.

# 5. Layer Stack-up Design

The stack-up defines how copper and dielectric layers are arranged through the board thickness. A good stack-up controls impedance, reduces EMI, and improves signal/power integrity.

## 5.1 Typical 4-layer stack-up

| Layer | Function |
| --- | --- |
| Top (L1) | Signal layer (component placement) |
| L2 | Ground plane |
| L3 | Power plane |
| Bottom (L4) | Signal layer |

This arrangement places signal layers adjacent to reference planes, giving controlled impedance and a continuous return path — critical for signal integrity.

## 5.2 Stack-up design guidelines

- Keep every signal layer adjacent to a solid reference plane (ground or power) to control impedance and minimise loop area.

- Pair power and ground planes closely together to form decoupling capacitance across the whole board.

- Keep the stack-up symmetric about the centre to avoid warping during lamination and thermal cycling.

- Route high-speed/critical signals on layers next to an uninterrupted ground plane.

- Increase layer count for higher component/pin density, controlled impedance requirements, or improved EMI performance.

# 6. Component Placement Guidelines

- Group components by function (power supply, analog, digital, RF) and physically separate noisy sections from sensitive ones.

- Place decoupling/bypass capacitors as close as possible to IC power pins to minimise loop inductance.

- Orient components consistently to simplify inspection and automated assembly.

- Keep high-speed and RF components close together to minimise trace length.

- Place connectors and user-interface components (switches, LEDs, displays) considering the enclosure and mechanical access.

- Consider thermal management — spread heat-generating components, keep them away from heat-sensitive parts, and provide airflow/heatsink clearance.

- Maintain clearance for mounting holes, board edges (keep-out zones), and panelisation (breakaway tabs/mouse-bites).

- Follow the natural signal flow (input → processing → output) to reduce crossing traces.

# 7. Routing (Trace Layout)

## 7.1 Trace width and current capacity

Trace width determines current-carrying capacity and resistive (I²R) heating. Wider/thicker traces and higher copper weight allow higher current for a given temperature rise (commonly estimated using IPC-2221 charts/equations).

| Factor | Effect on allowable current |
| --- | --- |
| Trace width | Wider trace → higher current capacity |
| Copper thickness (weight) | Thicker copper → higher current capacity |
| Allowed temperature rise | Higher allowed rise → higher current capacity |
| Internal vs. external layer | Internal traces carry less current than external for the same width (worse heat dissipation) |

## 7.2 Trace spacing and clearance

- Spacing between traces/pads must satisfy the fabricator's minimum clearance and the voltage-based creepage/clearance requirements (IPC-2221) to avoid arcing or shorts.

- High-voltage nets require larger spacing than low-voltage logic nets.

- Maintain clearance from board edge (typically ≥ 0.5–1 mm depending on fabricator) to avoid damage during routing/cutting.

## 7.3 Vias

| Via type | Description |
| --- | --- |
| Through-hole via | Drilled through the entire board, connects any layer |
| Blind via | Connects an outer layer to one or more inner layers, not through the whole board |
| Buried via | Connects inner layers only, not visible from either outer surface |
| Microvia | Very small laser-drilled via (used in HDI boards), typically ≤ 150 µm |
| Via-in-pad | Via placed directly inside a component pad, often filled/plated over; used for fine-pitch BGA |

## 7.4 General routing guidelines

- Route critical/high-speed signals first, with shortest, most direct paths.

- Avoid 90° trace corners — use 45° angles or curves to reduce reflections and etching issues; keep right angles for lower-frequency work non-critical.

- Avoid routing directly under crystal oscillators or sensitive analog circuitry.

- Keep differential pairs matched in length and consistently spaced for controlled differential impedance.

- Avoid crossing splits in the reference plane beneath high-speed signals (this breaks the return current path and increases EMI/crosstalk).

- Use ground stitching vias near signal vias that change layers, to give the return current a nearby path.

- Keep analog and digital ground separate where required, joining them at a single point (star-ground) to avoid noise coupling.

# 8. Power and Ground Planes

- A dedicated ground plane provides a low-impedance return path for all signals and reduces loop inductance, EMI, and crosstalk.

- Power planes distribute supply voltage with minimal IR drop and provide additional plane capacitance for high-frequency decoupling.

- Split planes (multiple voltage domains on one layer) must be planned carefully — avoid routing signals across a plane split.

- Decoupling capacitors should be placed close to IC power pins, with a range of values (e.g., 100 nF + 10 µF) to cover different frequency ranges, and connected with short, low-inductance vias.

- Thermal relief patterns (spokes) are used to connect through-hole pads to planes, easing soldering while maintaining electrical connection.

# 9. Signal Integrity and EMI/EMC

## 9.1 Controlled impedance

High-speed digital and RF signals require traces with a specific characteristic impedance (commonly 50 Ω single-ended, 90–100 Ω differential) to prevent reflections. Impedance depends on trace width, copper thickness, dielectric thickness, and dielectric constant, and is normally calculated with field-solver or IPC-2141 formulas for microstrip/stripline geometries.

## 9.2 Crosstalk

Crosstalk is unwanted coupling of signal energy between adjacent traces due to mutual capacitance and inductance. It is reduced by:

- Increasing spacing between parallel traces (the 3W rule: keep spacing ≥ 3× trace width for low coupling).

- Routing sensitive signals on layers adjacent to a solid ground plane.

- Minimising parallel run length between aggressor and victim traces.

- Adding ground traces/guard traces between sensitive signals where necessary.

## 9.3 EMI / EMC considerations

- Minimise loop area for high dI/dt and high-frequency current paths (keep signal and return paths close together).

- Use ground planes and proper stack-up to contain fields within the board.

- Place EMI filtering components (ferrite beads, common-mode chokes) near noise sources or board entry/exit points.

- Shield or separate high-frequency clocks/oscillators from I/O and cabling.

- Terminate transmission lines correctly (series or parallel termination) to avoid ringing and radiated emissions.

- Follow return-current path rules — avoid gaps in reference planes under signal traces.

## 9.4 Signal integrity issues summary

| Issue | Cause | Mitigation |
| --- | --- | --- |
| Reflection | Impedance mismatch | Controlled impedance routing, proper termination |
| Crosstalk | Coupled adjacent traces | Increase spacing, ground shielding, minimise parallel length |
| Ringing/overshoot | Fast edge rates, unmatched impedance | Series termination resistors, controlled slew rate |
| Ground bounce | Simultaneous switching noise on shared return path | Solid ground plane, adequate decoupling |
| EMI radiation | Large current loops, unshielded high-speed signals | Minimise loop area, proper stack-up, shielding |

# 10. Thermal Management

- Use copper pours/planes to spread heat away from hot components.

- Add thermal vias beneath power components to conduct heat to inner/bottom copper layers or a heatsink.

- Increase copper weight on layers carrying significant current to reduce resistive heating.

- Consider metal-core PCBs for high-power LED or power-electronics applications.

- Maintain adequate spacing/airflow around heat-generating components; consider heatsinks or thermal pads where needed.

- Account for thermal expansion mismatches (CTE) between component packages and the PCB substrate, especially for BGA/QFN parts.

# 11. Design Rules and DRC

Design Rule Check (DRC) software verifies the layout against a rule set before fabrication. Typical rules include:

| Rule | Typical value (standard fabrication) |
| --- | --- |
| Minimum trace width | 0.15 – 0.2 mm (6–8 mil) |
| Minimum trace spacing | 0.15 – 0.2 mm (6–8 mil) |
| Minimum via drill size | 0.2 – 0.3 mm |
| Minimum annular ring | 0.05 – 0.15 mm |
| Minimum board-edge clearance | 0.5 – 1 mm |
| Minimum solder mask clearance | 0.05 – 0.1 mm |

Note: exact values vary by PCB fabricator and process (standard vs. HDI/fine-pitch capability) — always confirm against the manufacturer's capability table.

## 11.1 Electrical Rule Check (ERC)

Performed at the schematic level before layout — checks for unconnected pins, conflicting outputs driving the same net, missing power/ground connections, and mismatched net classes.

# 12. Design for Manufacturing (DFM) and Assembly (DFA)

## 12.1 DFM guidelines

- Follow the fabricator's minimum trace/space and drill capabilities to avoid yield loss.

- Avoid acid traps (acute-angle copper features) that can cause etching defects.

- Maintain consistent copper density across the board to avoid uneven etching/plating.

- Use standard, widely-stocked drill sizes and via dimensions where possible.

- Provide fiducial markers for automated optical alignment during assembly.

- Panelise boards appropriately for automated assembly, with break-away tabs or v-scoring.

## 12.2 DFA guidelines

- Maintain adequate component-to-component spacing for pick-and-place and rework access.

- Keep consistent component orientation to simplify inspection.

- Avoid placing tall components near short ones where reflow oven airflow or stencil printing could be affected.

- Ensure polarity and pin-1 markings are clear on silkscreen for correct assembly.

- Design test points for in-circuit testing (ICT) and program/debug headers where required.

# 13. Footprints / Land Patterns

A footprint (land pattern) is the arrangement of copper pads that matches a component's physical package, defined by IPC-7351 standards for most SMD packages. Correct footprint design affects solder joint reliability, thermal performance, and manufacturability.

| Package type | Description |
| --- | --- |
| THT (Through-Hole Technology) | Leads inserted through drilled holes, soldered on opposite side; strong mechanical joint |
| SMD (Surface-Mount Device) | Leads/pads soldered directly onto the board surface; smaller, denser, suited to automated assembly |
| QFP / QFN | Quad flat package with leads on all four sides (QFP) or no leads (QFN, pads underneath) |
| BGA (Ball Grid Array) | Solder balls arranged in a grid under the package; high pin density, needs X-ray inspection |
| SOIC / SOT | Small-outline IC / transistor packages, common for discretes and small ICs |

# 14. High-Speed Design Considerations

- Treat a trace as a transmission line when its length approaches or exceeds a fraction of the signal's rise-time-related wavelength (rule of thumb: if trace length > 1/6 of the electrical length of the rising edge).

- Match lengths of differential pairs and bus signals to control timing skew.

- Use consistent, controlled-impedance routing for clocks, high-speed buses (e.g., USB, HDMI, DDR memory).

- Avoid stubs (unterminated branch traces) which cause reflections on high-speed nets.

- Provide adequate decoupling with low-ESL capacitors placed close to IC power pins.

- Follow the memory/interface vendor's layout guidelines (length matching, via count limits, reference plane continuity) for interfaces like DDR3/DDR4.

# 15. Testing and Design for Test (DFT)

- In-Circuit Test (ICT) — uses a bed-of-nails fixture to probe test points and verify component values/connections.

- Flying Probe Test — movable probes test nets without a dedicated fixture; suited to low-volume/prototype boards.

- Boundary Scan (JTAG) — tests interconnects on digital ICs supporting the IEEE 1149.1 standard, without physical probe access.

- Automated Optical Inspection (AOI) — camera-based inspection for solder defects, missing/misaligned components.

- X-ray inspection — used to inspect hidden solder joints, e.g., under BGA packages.

- Functional testing — verifies the assembled board performs its intended function under simulated operating conditions.

Exam tip: Include test points on nets that cannot be probed directly (e.g., under BGAs) — a common DFT question in exams.

# 16. Manufacturing Outputs

| File / output | Purpose |
| --- | --- |
| Gerber files (RS-274X) | Describe each copper layer, solder mask, silkscreen, and paste layer for fabrication |
| Drill file (Excellon) | Specifies hole locations and sizes for drilling |
| Pick-and-place (centroid) file | Component reference designator, position, rotation for automated placement |
| Bill of Materials (BOM) | List of all components, values, packages, and part numbers |
| IPC-2581 / ODB++ | Modern unified data formats combining fabrication and assembly data in one file |
| Assembly drawing | Shows component placement, orientation, and reference designators for manual/automated assembly |

# 17. Common PCB Design Software

| Tool | Notes |
| --- | --- |
| Altium Designer | Industry-standard professional PCB design suite |
| KiCad | Free, open-source, widely used in education and hobbyist/professional projects |
| Eagle (Autodesk) | Popular for hobbyist and small commercial designs |
| OrCAD / Allegro (Cadence) | Used in industry for complex and high-speed designs |
| Mentor Graphics (PADS/Xpedition, Siemens) | Used for complex, high-density enterprise designs |
| EasyEDA | Browser-based, popular for quick prototyping with integrated ordering |

# 18. Design Checklist (Quick Revision)

## Before sending to fabrication, verify:

- Schematic and layout match (net list verified, no unrouted nets).

- DRC and ERC pass with no unresolved violations.

- Stack-up and impedance targets confirmed with the fabricator.

- Decoupling capacitors placed close to all IC power pins.

- Ground/power planes are continuous under high-speed and sensitive signals.

- Trace widths sized for expected current with acceptable temperature rise.

- Adequate clearance from board edge, mounting holes, and between high-voltage nets.

- Silkscreen is legible, does not overlap pads, and shows correct polarity/pin-1 markers.

- Fiducials, tooling holes, and panelisation features included if required.

- Test points provided for critical/inaccessible nets.

- Thermal management addressed for power components.

- Gerber, drill, BOM, and pick-and-place files generated and cross-checked.

# 19. Quick-Reference Summary Table

| Concept | Key point to remember |
| --- | --- |
| PCB purpose | Mechanical support + reliable, repeatable electrical interconnection |
| Layer count | Higher layer count → higher density, better SI/EMI performance, higher cost |
| Stack-up rule | Every signal layer should be adjacent to a solid reference plane |
| Trace width | Set by required current-carrying capacity and impedance target |
| Vias | Through-hole (simplest), blind/buried/microvia (HDI, higher density) |
| Decoupling | Place capacitors as close as possible to IC power pins |
| Crosstalk control | 3W spacing rule, adjacent ground plane, minimise parallel run length |
| Return path | Never let return current cross a plane split under a signal trace |
| DFM | Design within the fabricator's minimum trace/space/drill capability |
| DFT | Add test points and boundary-scan support for inaccessible nets |